# Luxaflex ® Branded Specification Documents

The following branded section has been prepared by Construction Information Limited (Masterspec®) in conjunction with New Zealand Window Shades. The use of this section is subject to the following terms and conditions:

Users of this section shall rely solely on their own professional judgement and skill in determining the appropriateness of using or referring to this section and to satisfy themselves as to the accuracy of the data it contains.

Users acknowledge that neither Construction Information Limited nor the manufacturer of the products and systems described in the text are in any way liable in respect of any particular use or application of the material provided.

For more information and specification assistance please contact Luxaflex Customer Services on Free Phone 0800 223 224 to be directed to your local representative.

# 5531L	LUXAFLEX ROLLER BLINDS, SHADES AND SUNSCREENS

## 1.	GENERAL

This section relates to the supply and fixing of;

-	sunscreens, available as interior/exterior roller blinds, with chain, spring or automated systems.

-	rollershades, available with spring, chain or automated operating systems.

### 1.1	RELATED WORK

Refer to 5261L LUXAFLEX COUNTRYWOODS® SHUTTERS for basswood timber shutters

Refer to 5530L LUXAFLEX BLINDS, CURTAINS AND TRACKS for blinds, curtains, tracks

### 1.2	ABBREVIATIONS AND DEFINITIONS

Refer to the general section 1232 INTERPRETATION & DEFINITIONS for abbreviations and definitions used throughout the specification.

The following abbreviations apply specifically to this section:

BMS	Building Management System

HAS	Home Automation System

Documents

### 1.3	DOCUMENTS REFERRED TO

Refer to the general section 1233 REFERENCED DOCUMENTS.  The following documents are specifically referred to in this section:

NZBC C/AS1	Fire Safety

### 1.4	MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer's and supplier's documents referred to in this section are:

New Zealand Window Shades

Manufacturer/supplier contact details:

Company:	New Zealand Window Shades

Web:	www.luxaflex.co.nz

Telephone:	0800 223 224

Warranties

### 1.5	WARRANTY - MANUFACTURER/SUPPLIER

Provide a material manufacturer/supplier warranty:

2 years:	Commercial Installation - For all components, including operating mechanisms and controls.

3 years:	Domestic Installation - For all components, including operating mechanisms and controls.

-	Provide this warranty on the manufacturer/supplier standard form.

-	Commence the warranty from the date of installation.

Refer to the general section 1237 WARRANTIES for additional requirements.

Requirements

### 1.6	QUALIFICATIONS

Work to be carried out by trades people experienced, competent and familiar with the materials and techniques specified.

### 1.7	NO SUBSTITUTIONS

Substitutions are not permitted to any of the specified systems, components and associated products listed in this section.

### 1.8	SAMPLES

Provide the following samples for review by the Contract Administrator;

Colour range samples.

Manufacturers control systems for fully automated screens and shades.

### 1.9	INFORMATION FOR OPERATION AND MAINTENANCE

Provide the following general operation and maintenance information as electronic PDF format documents:

~

Provide this information prior to practical completion.

### 1.10	OPERATION AND MAINTENANCE MANUALS

Refer to the general section 1239 OPERATION & MAINTENANCE for the requirements for submission and review of operation and maintenance manuals.

Provide the following operation and maintenance manual(s):

~

Include the following in the operation and maintenance manual(s):

~

### 1.11	SPARES AND MAINTENANCE MATERIALS

Provide spares and maintenance materials listed in SELECTIONS to the Principal for ongoing maintenance requirements.

Performance

### 1.12	FIRE HAZARD

Do not provide materials which, when subject to fire conditions, will emit excessive smoke or dangerous fumes.

### 1.13	EARLY FIRE HAZARD PROPERTIES

Early fire hazard properties of internal surface materials/systems when tested to NZBC C/AS1, Appendix C, Test Methods, must not exceed the following indices to NZBC C/AS1, Table 6.2.  All fabrics comply with AS 1530.3

| Properties | Index | Item | Location |
| --- | --- | --- | --- |
| (FI) Flammability index | ~ | ~ | ~ |
| (SFI) Spread of flame index | ~ | ~ | ~ |
| (SDI) Smoke development index | ~ | ~ | ~ |

Quality control and assurance

### 1.14	INSPECTIONS

~

## 2.	PRODUCTS

Materials

### 2.1	SUNSCREENS

Available in chain, spring and automated operating systems.  All control options and automation utilise extruded aluminium roller tube and hardwearing UV resistant plastics.  Available in 43mm or heavy duty 60mm single or 43mm twin roller sets.

### 2.2	ROLLERSHADES

Available in chain, spring and automated operating systems.  All control options and

automation utilise extruded aluminium roller tube and hardwearing UV resistant plastics.

Available in single 43mm or heavy duty 60mm or 43mm twin roller sets.

Accessories

### 2.3	SOFT LIFT SPRING ASSIST

Supplied with all blinds 4m² and larger.  Optional for other blinds that are difficult to raise manually.  A spring mechanism is added to the end of the chain control inside the aluminium roller tube.

### 2.4	CASSETTE HEADBOX AND/OR SIDE GUIDES

The cassette system excludes all light at the top and sides of the blind.  The side guides fit to the side jamb or back of the jamb.

### 2.5	GUIDE WIRE TENSION SYSTEM

Stainless steel guide wire system that assists Rollershade and Sunscreen blinds to maintain a straight drop over heights of 2.5m or greater.  Fitted on inside or outside fitting blinds, complete with brackets top and bottom of the window reveal.  Stainless steel wire passes through specifically designed bottom rail end caps to keep the fabric in position.

### 2.6	AUTOMATION

Proprietary automation to single or groups of sunscreens and rollershades.  System includes electric motor, remote control or hard wired switch controls.  Can be integrated into building management systems (BMS) or Home Automation Systems (HAS).

## 3.	EXECUTION

Conditions

### 3.1	DELIVERY AND STORAGE

Handle shades, blinds, operating equipment and accessories to avoid damage.  Keep dry in transit, store clear of and on a level floor and cover for protection.  Avoid distortion, stretching, compression and damage to the materials.  Do not use damaged or wet materials.

### 3.2	PRE-INSTALLATION REQUIREMENTS

Ensure that conditions are suitable for the installation, including painting and flooring completed.  Arrange for the programming of the work to suit required practice.

Installation/application

### 3.3	INSTALLATION

Carry out the fixing, installation and fitting to finish rigid, plumb, true to line and face and square, to the manufacturer's requirements.

### 3.4	INSTALL COMPONENTS

Fix all components to the manufacturer's requirements.

Completion

### 3.5	ROUTINE CLEANING

Carry out routine trade cleaning of this part of the work including periodic removal of all debris, unused materials and elements from the site.

### 3.6	DEFECTIVE OR DAMAGED WORK

Replace damaged or marked elements where repair is not possible or will not be acceptable.  Adjust operation of equipment and moving parts not working correctly.

Leave work to the standard required for following procedures.

### 3.7	PROTECTION

Provide the following temporary protection of the finished work:

Protect from water, dust, and other destructive elements.

Commissioning

### 3.8	FINAL INSPECTION AND TESTING

~

## 4.	SELECTIONS

For further details on selections go to www.luxaflex.co.nz

Substitutions are not permitted to the following;

Materials

### 4.1	SUNSCREEN

Location:	~

Width:	~

Height:	~

Roller set number:	~

Colour:	~

Fabric:	~

### 4.2	ROLLERSHADE

Location:	~

Width:	~

Height:	~

Roller set number:	~

Colour:	~

Fabric:	~

Accessories

### 4.3	SOFT LIFT SPRING ASSIST

Location:	~

### 4.4	CASSETTE HEADBOX AND SIDE GUIDES

Location:	~

Headbox Length:	~

Headbox Height:	~

Headbox colour:	white

Side guide length:	~

Side guide colour:	white

### 4.5	GUIDE WIRE TENSION SYSTEM

Location:	~

### 4.6	AUTOMATION

Location:		~

Model:		~

Remote Model:		~

Number of channels:	~

Wall control model:		~

Number of wall controls:	~

Location of wall controls:	~

Spares and maintenance materials

### 4.7	SPARES AND MAINTENANCE MATERIALS

Provide the following spares and maintenance materials:

Item/quantity:	~

Deliver to:	~